CFD Simulation for Energy Systems

ANSYS Fluent Workflow

Project Task C — Gas–Solid Fluidized Bed Reactor

| Item | Detail |
| --- | --- |
| Course | CFD Simulation for Energy Systems, SS 2026 |
| Institution | Technische Universität München — Campus Straubing Professur für Regenerative Energiesysteme |
| Supervisors | Bernhard Huber, M.Sc.; Sahib Abdullayev, M.Sc. |
| Task | Project C — Fluidized Bed |
| Target software | ANSYS Fluent 2025 R2 (v252) |
| Physical model | Eulerian–Granular Two-Fluid Model (TFM) with KTGF closures |
| Source document | CFDSS2026ProjectUpdateFBR.pdf |

# Contents

# 1. Resolve These Two Points Before You Start

## 1.1 The documentation link may point at the wrong tutorial

The task sheet links to ansyshelp.ansys.com/…/v252/en/dem_tut/dem_tut_14.html. The dem_tut path is the Rocky DEM tutorial collection, not the Fluent tutorial guide (flu_tut).

However, sub-tasks (b), (d), (e), (f) and (g) describe Fluent’s Eulerian–Granular model unambiguously: a granular secondary phase, k-ε turbulence on the gas phase, a patched initial volume fraction, and a comparison against the Ergun correlation. These are Two-Fluid Model concepts, not DEM concepts.

Reinforcing the ambiguity: the Details block asks you to pick dₚ “for a reasonable total number of particles.” In a 2D Eulerian TFM there is no particle count — the solids are a continuum. That constraint only binds a DEM run. See Section 10 for the particle-count arithmetic.

Action required Ask Huber / Abdullayev which model is intended. This document covers the Fluent Eulerian–Granular path, which satisfies every sub-task exactly as written. Section 10 covers the Rocky DEM variant if that turns out to be the intent.

## 1.2 Confirm your group’s assigned regime

Sub-task (e) requires at least one of Bubbling, Turbulent, or Fast fluidized, per the Excel file on Moodle. This determines your inlet velocity and — critically — your particle diameter. See Section 3 before committing to dₚ.

# 2. Reference Calculations

Given: ρₚ = 1500 kg/m³, ρg = 1.2 kg/m³, μ = 1.8×10⁻⁵ Pa·s, εmf = 0.40 (αₚ = 0.60), spherical particles (ϕₛ = 1), g = 9.81 m/s².

## 2.1 Minimum fluidization velocity (sub-task c)

Umf is the superficial gas velocity at which the upward drag force on the bed exactly balances its net (buoyant) weight. Below Umf the bed is a fixed packed bed and Δp rises with U. At U_mf the particles are just suspended: Δp stops rising, plateaus at the bed weight per unit area, and stays there for all higher velocities.

The working equation is obtained by setting the Ergun packed-bed pressure gradient equal to the buoyant bed weight per unit height:

1.75/(ε_mf³ ϕₛ) · Re_mf²  +  150(1−ε_mf)/(ε_mf³ ϕₛ²) · Re_mf  =  Ar

Ar    = dₚ³ ρ_g (ρₚ − ρ_g) g / μ²

Re_mf = dₚ U_mf ρ_g / μ

Terminal velocity U_t is from Haider & Levenspiel (1989) for spheres.

| dₚ [mm] | Ar | Re_mf | U_mf [m/s] | U_t [m/s] | Re_t |
| --- | --- | --- | --- | --- | --- |
| 1 | 5.45×10⁴ | 25.8 | 0.387 | 5.24 | 349 |
| 2 | 4.36×10⁵ | 103.1 | 0.773 | 8.01 | 1068 |
| 3 | 1.47×10⁶ | 207.6 | 1.038 | 10.01 | 2001 |
| 4 | 3.49×10⁶ | 332.2 | 1.246 | 11.66 | 3108 |
| 5 | 6.81×10⁶ | 473.9 | 1.422 | 13.09 | 4364 |

Self-consistency check at dₚ = 3 mm, U = Umf = 1.038 m/s. Ergun gives 1752 Pa/m (viscous) + 7070 Pa/m (inertial) = **8822 Pa/m**, and the required gradient (ρₚ − ρg)(1 − ε)g = 1498.8 × 0.6 × 9.81 = 8822 Pa/m. Exact match, as it must be.

Cross-check with Wen & Yu (1966), Remf = √(33.7² + 0.0408 Ar) − 33.7: at dₚ = 3 mm this gives Umf = 1.068 m/s, i.e. +3 % against the Ergun-balance value. Report both — the agreement between two independent correlations is a cheap and genuine validation point for sub-task (c).

## 2.2 The fluidized pressure-drop plateau

Δp_plateau = (ρₚ − ρ_g)(1 − ε) g H₀

= 1498.8 × 0.60 × 9.81 × 0.40

= 3529 Pa

Your single most important validation target Δp ≈ 3529 Pa, independent of dₚ and of U₀ (above U_mf). If your simulated bed pressure drop does not settle near 3.5 kPa, something is wrong with the patch, the boundary conditions, or the packing limit — not with the physics.

## 2.3 A note on particle classification

At ρₚ = 1500 kg/m³ and dₚ = 1–5 mm these are Geldart Group D particles (large, dense, spoutable) — not Group B. Group D beds spout and channel readily and produce coarser bubbles than Group B. State the classification in your report; it explains most of what you will see in the contours.

# 3. Choosing dₚ and U₀ for Your Regime

Regime boundaries scale with Umf and Ut from Section 2. The values below are established rules of thumb (Kunii & Levenspiel 1991; Bi & Grace 1995 for the turbulent onset), not exact thresholds — treat them as targets, and confirm the regime from your own contours.

| Regime | Criterion | dₚ = 3 mm | dₚ = 1 mm |
| --- | --- | --- | --- |
| Fixed bed | U₀ < U_mf | < 1.04 m/s | < 0.39 m/s |
| Bubbling | ≈ 2–4 × U_mf | 2.0 – 4.0 m/s | 0.8 – 1.5 m/s |
| Turbulent | ≈ Uc, roughly 4–7 × Umf | 5.0 – 7.0 m/s | 1.8 – 2.7 m/s |
| Fast fluidized | U₀ > U_t, needs solids recirculation | > 10.0 m/s | > 5.2 m/s |

## 3.1 Recommendation by assignment

- Bubbling — dₚ = 3 mm, U₀ ≈ 2.5 m/s (≈ 2.4 U_mf). Comfortable, and bubbles are well resolved on the mesh of Section 4.

- Turbulent — dₚ = 3 mm, U₀ ≈ 6 m/s. Workable without changes.

- Fast fluidized — use dₚ = 1 mm, U₀ ≈ 6 m/s.

If your group drew fast fluidization, do not use dₚ = 3–5 mm At 3 mm you need U₀ > 10 m/s in a 0.28 m column. Transit time through the 1 m column drops to ≈ 0.1 s, the CFL-limited time step collapses, and without a recycle loop the bed simply blows out of the top within a second. At dₚ = 1 mm (Umf = 0.387 m/s, Ut = 5.24 m/s) an inlet of ≈ 6 m/s is sufficient and the case is tractable. The Details block permits 1–5 mm; the 3–5 mm in sub-task (b) is a suggestion, and sub-task (i) explicitly allows deviation provided it is reported. Either add a solids recycle/return, or accept that you are simulating a transient blow-out — and say so explicitly in the report.

Whatever you choose, sub-task (i) requires you to state the exact values used. Put a parameter table at the front of your report.

# 4. Geometry and Mesh (sub-task a)

Work in 2D planar unless you have a specific reason not to A 2D bed is standard practice for TFM fluidization studies, runs roughly two orders of magnitude faster, and is what makes the velocity sweep of Section 7 feasible at all. Note in the report that 2D over-predicts bubble size somewhat and suppresses the third velocity component.

- Geometry. In DesignModeler or SpaceClaim, create a rectangle 0.28 m (x) × 1.0 m (y).

  - If the tutorial geometry imports in millimetres, use Domain → Mesh → Scale → Convert Units mm → m in Fluent.

  - Verify with Domain → Mesh → Info → Size that the bounding box reads 0.28 × 1.0.

- Named selections. Create these in the mesher, not afterwards:

  - inlet — bottom edge (y = 0)

  - outlet — top edge (y = 1.0)

  - wall-left, wall-right — the two vertical edges

- Mesh. Uniform quadrilateral, element size 4 mm → 70 × 250 = 17,500 cells.

  - Use a mapped/structured quad control (Mesh → Method → Quadrilateral Dominant, or Multizone / Face Meshing).

  - Check orthogonal quality > 0.9 — trivially satisfied for a uniform quad mesh.

## 4.1 Mesh sizing rationale (and a free mesh-independence study)

TFM closures assume the cell is large compared with a particle but small compared with a bubble. Target Δx ≈ 2–10 dₚ.

At dₚ = 3 mm, a 4 mm cell gives Δx/dₚ = 1.3 — slightly below the ideal range. So also run an 8 mm mesh (35 × 125 = 4,375 cells, Δx/dₚ = 2.7) and compare bed expansion and time-averaged Δp. This gives you a defensible mesh-independence study at almost no extra cost, since the coarse case runs in a fraction of the time.

# 5. Fluent Setup

Launch Fluent in 2D, Double Precision, 4 cores. Double precision matters here — the volume fraction near the packing limit of 0.63 is sensitive to round-off.

## 5.1 General (sub-task a)

- Solver: Pressure-Based, Transient, Planar, Absolute velocity formulation.

- Gravity: on, Y = −9.81 m/s².

The most common single failure in this project Forgetting gravity. The bed then just sits there and nothing fluidizes, no matter what inlet velocity you set. Check it before every run.

## 5.2 Models (sub-task b)

- Multiphase → Eulerian, Number of Eulerian Phases = 2. Leave the Dense Discrete Phase Model off.

- Viscous → k-epsilon (2 eqn), Standard, Standard Wall Functions.

  - Multiphase Turbulence: Mixture — the robust default, one shared k-ε field.

  - To satisfy the task wording “k-ε turbulence for gas phase” literally, select Per Phase instead and enable turbulence only on air. It is less stable and roughly 30 % slower. Mixture is accepted practice for dense beds — state your choice and the reason.

  - Do not use Dispersed — it assumes a dilute secondary phase, which αₚ = 0.6 flatly violates.

- Energy: off. See Section 8 on what “particle temperature” means here.

## 5.3 Materials

- air: ρ = 1.2 kg/m³ (constant, not ideal-gas), μ = 1.8e-5 kg/m-s.

- New solid material sand: ρ = 1500 kg/m³. With the energy equation off, only density matters.

## 5.4 Phases (sub-task b)

- Phase 1 (primary) — name air, material air.

- Phase 2 (secondary) — name solids, material sand, Granular = on, with:

| Setting | Value | Note |
| --- | --- | --- |
| Diameter | 0.003 m | or your chosen dₚ |
| Granular Viscosity | Gidaspow | — |
| Granular Bulk Viscosity | Lun et al. | — |
| Frictional Viscosity | Schaeffer | dense-packing stress |
| Frictional Pressure | Based-ktgf | — |
| Friction Packing Limit | 0.61 | onset of frictional regime |
| Angle of Internal Friction | 30° | — |
| Solids Pressure | Lun et al. | — |
| Radial Distribution | Lun et al. | — |
| Granular Temperature Model | Phase Property → Algebraic | switch to PDE for the sub-task (f) plot — see §8 |
| Packing Limit | 0.63 | random close packing |

- Phase Interaction → Drag: `gidaspow`. Gidaspow blends Ergun (dense, α_g < 0.8) with Wen–Yu (dilute) — exactly right for a bed that spans both. Syamlal–O’Brien is the usual alternative; running one case with each and comparing bed expansion is a good sub-task (j) extra.

- Restitution coefficient (air–solids and solids–solids): 0.9.

## 5.5 Boundary conditions (sub-task d)

| Zone | Type | Settings |
| --- | --- | --- |
| inlet | velocity-inlet | Mixture tab: Turbulent Intensity 5 %, Hydraulic Diameter 0.28 m air tab: Velocity Magnitude = U₀, normal to boundary solids tab: Velocity = 0, Volume Fraction = 0 |
| outlet | pressure-outlet | Gauge Pressure = 0 Pa Backflow: solids Volume Fraction = 0, Turb. Intensity 5 %, D_h = 0.28 m |
| wall-left wall-right | wall | air: No Slip solids: No Slip (as specified in the task) |

On the solids wall boundary condition The task specifies no-slip, so use it. Physically, Johnson–Jackson partial slip (specularity coefficient ≈ 0.2–0.5) is more realistic and is what the literature uses: no-slip over-damps the near-wall solids and slightly under-predicts bed expansion. One comparison run and a short paragraph makes a good sub-task (j) extra.

## 5.6 Initialization and patching the packed bed (sub-task d)

This is the step most groups get wrong. The order matters.

- Define the bed region first. Use Domain → Adapt → Cell Registers → New → Region (not Refinement Criteria):

  - Shape: Hex — works as a rectangle in 2D

  - X min = 0, X max = 0.28, Y min = 0, Y max = 0.40

  - Name it bed, then Save/Display and confirm the highlighted region is the bottom 40 % of the column.

- Standard Initialize. Solution → Initialization → Standard — not Hybrid, which does not handle multiphase volume fractions well. Compute From: inlet. Ensure solids Volume Fraction = 0, then Initialize.

- Patch. Solution → Initialization → Patch:

  - Phase: solids  |  Variable: Volume Fraction  |  Value: 0.6

  - Registers to Patch: bed, then click Patch.

- Verify before running. Plot a contour of solids Volume Fraction (Filled, Node Values off). You must see a sharp block of 0.6 from y = 0 to y = 0.40 and 0 above. If it is smeared or empty, the patch failed — repeat step 3.

## 5.7 Solution methods

- Scheme: Phase Coupled SIMPLE

- Gradient: Least Squares Cell Based

- Momentum: First Order Upwind for the first ≈ 0.5 s, then Second Order Upwind

- Volume Fraction: First Order Upwind initially, then QUICK once running — QUICK preserves the bed interface far better; first-order smears it badly

- Turbulent KE / Dissipation: First Order Upwind → Second Order Upwind

- Transient Formulation: First Order Implicit — bounded and stable for volume fraction. Second Order Implicit is more accurate but more prone to unbounded αₛ; try it only once the case runs cleanly.

## 5.8 Under-relaxation factors

Start conservative; relax once the case is stable.

| Variable | Start | Once stable |
| --- | --- | --- |
| Pressure | 0.3 | 0.5 |
| Momentum | 0.2 | 0.4 |
| Volume Fraction | 0.2 | 0.5 |
| Granular Temperature | 0.2 | 0.2 |
| Turbulent KE / Dissipation | 0.5 | 0.8 |

Residual Monitors: set all Absolute Criteria to 1e-3 — this is the task’s stated minimum convergence criterion. Keep “Check Convergence” ticked.

## 5.9 Run controls

- Time Step Size: 5e-4 s. If you see divergence or floating-point errors in the first 100 steps, drop to 1e-4 s for 0.2 s, then step back up.

- Max Iterations / Time Step: 40. If Fluent routinely hits 40 without reaching 1e-3, your time step is too large.

- Number of Time Steps: enough for 12–20 s of physical time — at dt = 5e-4 that is 24,000–40,000 steps.

- Budget: on 4 cores, a 17,500-cell 2D case takes roughly 3–8 hours for 15 s of physical time. Start it overnight.

- Reporting Interval: 1. Profile Update Interval: 1.

# 6. Data Collection During the Run

Set all of this up before you press Calculate Re-running a 15 s case because you forgot a monitor is the single largest time sink in this project.

## 6.1 Pressure-drop report (sub-tasks e, g)

Solution → Report Definitions → New → Surface Report → Area-Weighted Average

- Field Variable: Pressure → Static Pressure; Phase: mixture

- Surface: inlet  |  Name: dp_bed

- Tick Report File and Report Plot, and set Create Output Parameter.

Because the outlet sits at 0 Pa gauge, dp_bed is the pressure drop across the bed, plus a negligible dilute-freeboard contribution. Compare it against the 3529 Pa target from Section 2.

## 6.2 Bed inventory monitor

New → Volume Report → Volume Integral of solids Volume Fraction over the fluid domain. This should stay constant — it is your mass-conservation check. If it drifts downward, solids are leaving through the outlet, which is expected only in fast fluidization.

## 6.3 Time-averaging (sub-task f — essential)

Solution → Calculation Activities → Data Sampling for Time Statistics

- Enable it only after the startup transient has died, at roughly 3 s.

- Practical method: run 3 s with sampling off, pause, tick sampling on, then run to 15 s. You get statistics over 12 s of developed flow.

- Sampling Interval: 1.

- This creates Mean Static Pressure, Mean Volume Fraction and Mean Velocity fields that you can contour directly.

## 6.4 Autosave and image capture

Sub-task (f) asks for plots at the start, middle and end of the simulation, so:

- Solution → Calculation Activities → Autosave — Save Data File Every 1000 time steps (every 0.5 s). You will want these for post-hoc plotting.

- Set up Solution Animations on a contour of solids Volume Fraction, writing to file every 100 steps. This gives you a bubble-formation movie, which directly answers “where do bubbles form and how do they coalesce?”

# 7. Velocity Sweep and the Ergun Comparison (sub-task g)

You need Δp versus U₀ spanning both below and above U_mf.

Do not ramp velocity within a single transient run The bed exhibits hysteresis — a defluidizing bed follows a different Δp path than a fluidizing one. A ramp will contaminate the curve. Use separate runs.

Method: 8–11 separate short runs. For dₚ = 3 mm (U_mf = 1.038 m/s):

U₀ = 0.2, 0.4, 0.6, 0.8, 0.9, 1.0, 1.1, 1.3, 1.6, 2.0, 2.5   [m/s]

For each point: initialize and patch fresh, run 2.0 s, and time-average dp_bed over the last 1.0 s. Points below U_mf reach steady state almost immediately — it is a fixed bed. Points above need the full 2 s for the bubbling to become statistically stationary.

Automate this with a Fluent journal so you are not clicking for hours: write a .jou that loops over the velocity list, patches, solves, and writes each report file.

## 7.1 The Ergun comparison

For the packed-bed branch (U₀ < U_mf), compare against:

Δp/L = 150 μ (1−ε)² U₀ / (ε³ dₚ²)  +  1.75 ρ_g (1−ε) U₀² / (ε³ dₚ)

with L = H₀ = 0.40 m,  ε = 0.40,  dₚ = 0.003 m

A physical point most groups will miss The two Ergun terms are viscous (linear in U₀) and inertial (quadratic). At Rep ≈ 200, the **inertial term dominates**: at Umf it contributes 7070 of the 8822 Pa/m, i.e. 80 %. This is a direct consequence of working with coarse Geldart Group D particles rather than fine ones. Say so explicitly in the report.

## 7.2 Expected curve shape

A quadratic rise following Ergun up to U_mf ≈ 1.04 m/s, then a flat plateau at ≈ 3529 Pa for all higher velocities. A modest overshoot right at the transition is physical — the bed must first break its packed structure — and is worth pointing out if you capture it.

Use postproc/plot_pressure_drop.py in the repository for the plotting (currently an empty placeholder).

# 8. “Particle Temperature” in Sub-task (f)

The task asks you to plot particle temperature. There are two things this could mean, and they are completely different.

## 8.1 Granular temperature Θₛ — almost certainly what is meant

Θₛ [m²/s²] is the kinetic-theory measure of particle velocity fluctuation — the granular analogue of thermodynamic temperature. It comes free with the Eulerian–Granular model, needs no energy equation, and is the standard diagnostic for a fluidized bed. High Θₛ marks the vigorous, dilute bubble wakes; low Θₛ marks the dense emulsion phase.

- To get a proper transport field, set Granular Temperature Model = Partial Differential Equation in the solids phase panel (Section 5.4).

- Contour it via Contours → Phases… → Granular Temperature, Phase = solids.

## 8.2 Thermal temperature T — probably not

This would require enabling the Energy equation, a heat-transfer coefficient model (Gunn is standard for packed and fluidized beds), inlet and wall thermal boundary conditions, and a solids heat capacity. The task sheet specifies no thermal boundary conditions and no heat source at all — so with a uniform 300 K everywhere, nothing happens and the plot is a flat field.

Recommendation Plot granular temperature, and add one sentence stating which interpretation you used and why. If you want to hedge, enable Energy with a hot inlet (e.g. 400 K gas into a 300 K bed) as an “own test” under sub-task (j). That gives you a genuinely interesting thermal-front plot and covers both readings of the question.

# 9. Written Answers for Sub-task (h)

## 9.1 One-way, two-way and four-way coupling

The classification is due to Elghobashi (1994) and is based on the solids volume fraction αₛ:

| Coupling | αₛ range | What is modelled | Typical applications |
| --- | --- | --- | --- |
| 1-way | αₛ < 10⁻⁶ | Fluid moves the particles. Particles do not affect the fluid. | Dilute tracers, aerosols, spray dispersion |
| 2-way | 10⁻⁶ – 10⁻³ | Momentum exchanged both ways — particle drag appears as a source term in the fluid momentum equation. Particles still do not see each other. | Dilute sprays, pneumatic transport, particle-laden jets |
| 4-way | > 10⁻³ | 2-way plus particle–particle and particle–wall collisions and contacts. | Dense flows: fluidized beds, hoppers, packed beds, risers |

Your bed at αₛ = 0.60 is firmly four-way. This is precisely why the granular phase requires KTGF closures — solids pressure, granular viscosity, and the radial distribution function are the continuum representation of those particle–particle collisions.

## 9.2 ANSYS Rocky (DEM) versus Fluent (Eulerian–Granular)

| Aspect | Fluent Eulerian–Granular (TFM) | Rocky DEM |
| --- | --- | --- |
| Frame | Eulerian — solids as an interpenetrating continuum on the same mesh as the gas | Lagrangian — every particle tracked individually |
| Governing equations | PDEs (mass and momentum per phase), solved by the finite volume method | Newton–Euler ODEs per particle, integrated explicitly |
| Collisions | Modelled through KTGF closures: solids pressure, granular viscosity | Resolved through contact laws (Hertz–Mindlin, linear spring–dashpot) with real stiffness, damping and friction |
| Particle shape | Spheres only, single diameter, or a few discrete size bins | Arbitrary — polyhedra, fibres, flexible and breakable particles |
| Size distribution | Awkward — needs one additional Eulerian phase per size class | Native polydispersity |
| Cost scaling | Independent of particle count; scales with cell count | Scales with particle count; ≈ 10⁵–10⁷ particles feasible on GPU |
| Industrial scale | Yes | Only for coarse particles or small domains |
| Coupling | Inherently four-way through the closures | Inherently four-way, resolved directly; couples to Fluent one-way, two-way, or four-way with volume displacement |

In one sentence: the TFM trades particle-level fidelity for scalability. It cannot tell you contact forces, segregation by shape, attrition, or what any individual particle did — but it will run a full industrial reactor. DEM gives you all of that, but its cost scales with particle count, so you are confined to laboratory scale or coarse particles.

## 9.3 Why not use finite volume methods for particle modelling?

FVM is a continuum discretization. It integrates conservation PDEs over control volumes and requires the field variables to be well-defined continuum quantities across each cell. That fails for particles on three counts.

- The governing equations are the wrong type. A particle obeys Newton–Euler ODEs — m dv/dt = ΣF and I dω/dt = ΣT, six ODEs per particle, coupled through discrete, instantaneous, non-smooth contact events. There is no PDE to discretize over a mesh.

- Contacts are discontinuous. Collisions produce impulsive force changes over microsecond timescales. FVM’s implicit time-marching and flux-based spatial discretization are built for smooth fields, not for the non-smooth, event-driven mechanics of contact.

- Resolving particles with FVM is prohibitively expensive. It can be done — particle-resolved DNS with immersed boundaries — but it needs roughly 20 or more cells across each particle diameter, plus moving and deforming boundaries. For your bed that is 10⁵–10⁶ particles × ≈ 10⁴ cells each, i.e. 10⁹–10¹⁰ cells with moving interfaces: a research-grade supercomputer run for a few seconds of physics.

So the field splits cleanly: FVM for the fluid, which is a genuine continuum, and for the solids either DEM (track each particle, resolve the contacts) or a continuum granular closure (TFM with KTGF — treat the solids as a fluid and model the collisions). Fluent’s Eulerian–Granular model takes the second route; Rocky takes the first.

# 10. If the Rocky DEM Path Is Intended

The “reasonable total number of particles” hint in the task Details makes sense only here. At αₚ = 0.60, H₀ = 0.40 m and a 0.28 m width, the particle counts are:

| Domain | dₚ = 3 mm | dₚ = 5 mm |
| --- | --- | --- |
| Full 3D (0.28 × 0.40 × 0.28 m) | ≈ 1,330,000 | ≈ 287,000 |
| Thin slab (0.28 × 0.40 × 0.02 m) | ≈ 95,000 | ≈ 20,500 |

A full 3D bed at 3 mm is a million-particle DEM run — heavy even on a GPU. Use a thin quasi-2D slab, 20–30 mm deep, with periodic or frictionless front and back walls. At 20,000–95,000 particles the case is very comfortable for Rocky on a single GPU, and the slab geometry is standard practice for validating fluidized-bed DEM against 2D experimental beds.

In that configuration Fluent supplies the gas phase and Rocky the particles, coupled two-way with volume displacement — which is effectively four-way overall, since Rocky resolves the particle–particle contacts itself.

# 11. Execution Checklist

- Confirm the assigned regime from the Moodle Excel file — §1.2

- Confirm Fluent TFM versus Rocky DEM with the supervisors — §1.1

- Hand-compute U_mf for your chosen dₚ and write it up (sub-task c) — §2.1

- Build the 0.28 × 1.0 m geometry and verify the scale is in metres (sub-task a) — §4

- Mesh at 4 mm; also mesh at 8 mm for the mesh-independence study — §4.1

- Set transient solver and gravity Y = −9.81 (sub-task a) — §5.1

- Eulerian two-phase, k-ε, granular solids with Gidaspow drag (sub-task b) — §5.2–5.4

- Boundary conditions: velocity inlet, pressure outlet, no-slip walls (sub-task d) — §5.5

- Initialize, patch αₛ = 0.6 up to y = 0.40, and visually verify the patch (sub-task d) — §5.6

- Set up the dp_bed report, data sampling and autosave before running — §6

- Run 15 s of the assigned regime (sub-task e) — §5.9

- Plot time-averaged αₛ, gas velocity vectors and granular temperature at start / middle / end (sub-task f) — §6.3, §8

- Velocity sweep; plot Δp versus U₀ against Ergun; check the 3529 Pa plateau (sub-task g) — §7

- Write up coupling regimes, Rocky versus Fluent, and why not FVM (sub-task h) — §9

- Parameter table listing every value actually used (sub-task i) — §3

- Optional extras: Syamlal–O’Brien drag, partial-slip walls, thermal front (sub-task j)

# 12. Troubleshooting

| Symptom | Cause | Fix |
| --- | --- | --- |
| Bed never moves | Gravity not enabled | §5.1 — set Y = −9.81 |
| Bed falls through the bottom | Gravity sign wrong (+9.81) | §5.1 |
| Δp ≈ 0 | Patch failed; no solids in the domain | Redo §5.6; contour to verify |
| Δp far below 3529 Pa | Patched region wrong height, or αₛ patched below 0.6 | Check the register: Y max = 0.40 |
| Diverges within the first 50 steps | Time step too large, under-relaxation too high | dt → 1e-4; momentum URF → 0.2 |
| αₛ exceeds 0.63, or floating-point error | Packing limit or frictional model misconfigured | Verify packing limit 0.63, frictional packing limit 0.61, URF(VF) = 0.2 |
| Solids leave through the outlet | U₀ above U_t, or backflow VF ≠ 0 | Set backflow solids VF = 0; check U₀ against §2.1 |
| Bed interface smears into mush | Volume fraction still on First Order Upwind | Switch to QUICK — §5.7 |
| Hits 40 iterations every time step | Time step too large for the flow | Halve dt |
| Time-averaged plots identical to instantaneous | Data Sampling never enabled | §6.3 |

# References

Ergun, S. (1952). Fluid flow through packed columns. Chemical Engineering Progress, 48(2), 89–94.

Wen, C. Y. & Yu, Y. H. (1966). A generalized method for predicting the minimum fluidization velocity. AIChE Journal, 12(3), 610–612.

Lun, C. K. K., Savage, S. B., Jeffrey, D. J. & Chepurniy, N. (1984). Kinetic theories for granular flow. Journal of Fluid Mechanics, 140, 223–256.

Haider, A. & Levenspiel, O. (1989). Drag coefficient and terminal velocity of spherical and nonspherical particles. Powder Technology, 58(1), 63–70.

Kunii, D. & Levenspiel, O. (1991). Fluidization Engineering, 2nd ed. Butterworth-Heinemann.

Elghobashi, S. (1994). On predicting particle-laden turbulent flows. Applied Scientific Research, 52, 309–329.

Gidaspow, D. (1994). Multiphase Flow and Fluidization. Academic Press.

Bi, H. T. & Grace, J. R. (1995). Flow regime diagrams for gas–solid fluidization and upward transport. International Journal of Multiphase Flow, 21(6), 1229–1236.

ANSYS Fluent Theory Guide, Chapter 17 (Multiphase Flows) — Eulerian model and KTGF closures.